Computer System Architecture

Tutorial 2:

- A benchmark program is run on an 80 MHz processor. The executed program consists of 100,000 instruction execution, with the following instruction mix and clock cycle count:

| Instruction Type | Instruction Count | Cycle per Instruction |
| --- | --- | --- |
| Integer Arithmetic | 45000 | 1 |
| Data Transfer | 32000 | 2 |
| Floating Point | 15000 | 2 |
| Control Transfer | 8000 | 2 |

Determine the effective CPI, MIPS rate, and execution time for this program

- The performance of a 100MHz microprocessor P is measured by executing 10,000,000 instruction of benchmark code, which is found to take 0.25s. What are the values of CPI and MIPS for this performance experiment? Is P likely to be superscalar?

- Suppose that a single-chip microprocessor P operating at clock frequency of 50MHz is replaced by a new model P’, which has the same architecture as P but has a clock frequency of 75MHz.

  - If P has a performance rating of p MIPS for a particular benchmark program Q, what is the corresponding MIPS rating p’ for P

  - P takes 250s to execute Q in a particular personal computer system C. On replacing P by P’ in C, the execution time of Q drops only to 220s. Suggest a possible reason for this disappointing performance improvement.

- Suppose we have two implementations of the same instruction set architecture.   Computer A has a clock cycle time of 250 ps and a CPI of 2.0 for a given benchmark program, and computer B has a clock cycle time of 500 ps and a CPI of 1.2 for the same program.   Which computer is faster for this program and by how much?

- A compiler designer is deciding between two codes for a particular machine.  Based on the hardware implementation, there are three different classes of instructions:  Class A, Class B, and Class C, and they require one, two, and three cycles respectively.

First code has 5 instructions:   2 of A, 1 of B, and 2 of C.

Second code has 6 instructions:  4 of A, 1 of B, and 1 of C.

Which code is faster?

By how much?

What is the CPI for each code?