# Tan Chee Keat 25006123

Tutorial 4: WRES1201 Computer System Architecture

- What is the general relationship among access time, memory cost and capacity?

- Faster access time results in a greater cost per bit.

- Greater capacity allows for a smaller cost per bit.

- However, greater capacity also results in a slower average access time

- What are the differences among direct mapping, associative mapping and set associative mapping?

- Direct Mapping: Each block of main memory maps to only one unique cache line.

- Associative Mapping: A main memory block can be loaded into any line of the cache.

- Set Associative Mapping: A compromise where the cache is divided into sets, each containing a number of lines. A main memory block maps to one unique set, but it can be placed into any line within that set

- Consider a machine with a byte addressable main memory of 216 bytes and block size of 8 bytes. Assume that a direct mapped cache consisting of 32 lines is used with this machine.

  - How is a 16-bit memory address divided into tag, line number, and byte number?

  - Number of addressable units=216, thus address length=16

  - Block size is 8 bytes=23 bytes, thus word identifier, w=3

  - Number of line in cache =32 lines=25, thus line identifier, r=5

  - Tag=16-3-5=8

  - Into what line would bytes with each of the following addresses be stored?

    - 0001 0001 0001 1011

    - 1100 0011 0011 0100

    - 1101 0000 0001 1101

    - 1010 1010 1010 1010

  - i. line identifier=00011, line 3

  - ii. line identifier=00110, line 6

  - iii. line identifier=00011, line 3

  - iv. line identifier=10101, line 21

  - Suppose the byte with address 0001 1010 0001 1010 is stored in the cache. What are the addresses of the bytes stored along with it?

  - Since the block size is 8 bytes, the block spans the last 3 bits ranging from 000 to 111. The bytes stored alongside it cover the addresses 0001 1010 0001 1000 through 0001 1010 0001 1111.

  - How many total bytes of memory can be stored in the cache?

  - The cache holds 32 lines containing 8 bytes each, thus total capacity is 32x8=256 bytes.

  - Why is the tag also stored in the cache?

  - The tag uniquely identifies a memory block because no two blocks sharing the same cache line have the same tag field. The tag is checked against the memory address to verify if the correct data block is currently present in the cache.

- A cache has 64KB capacity, 128byte per lines and is 4-way set-associative. The system containing the cache uses 32-bit addresses. How many lines and sets does the cache have?

- Number of lines=(64x1024byte)/128bytes=512 lines

- Number of sets=512/4way=128 sets

- Describes three method used by cache memory to do replacement policy?

- Least Recently Used (LRU): Replaces the block in the cache set that has been there the longest with no reference to it.

- First-in-first-out (FIFO): Replaces the block in the cache set that has been there the longest, regardless of reference frequency.

- Least Frequently Used (LFU): Replaces the block in the set that has experienced the fewest total references

- What are the differences between write-through and write-back?

- Write-through: All write operations immediately update both the main memory and the cache. It is simple but generates substantial memory traffic, potentially creating a bottleneck.

- Write-back: Updates are initially made only to the cache, which minimizes memory writes. However, it requires complex circuitry because main memory may contain invalid data until the cache block is eventually swapped out.

- Consider a direct mapping single-level cache with an access time of 2.5ns, a line size of 64 bytes, and a hit ration of H= 0.95. Main memory uses a block transfer capability that has first-word (4 bytes) access time of 50ns and an access time of 5 ns for each word thereafter.

  - What is the access time when there is a cache miss? Assume that the cache waits until the line has been fetched from main memory and then re-executes for a hit.

  - Number of words for a line=64bytes/4bytes=16words

  - Total access time to fetch the words=50+(15x5)+2.5=127.5ns

  - Access time=(2.5x0.95)+(127.5x0.05)=8.75ns

  - Suppose that increasing the line size to 128 bytes increases the H to 0.97. Does this reduce the average memory access time?

  - Number of words for a line=128bytes/4bytes=32words

  - Total access time to fetch the words=50+(31x5)+2.5=207.5ns

  - Access time=(2.5x0.97)+(207.5x0.03)=8.65ns

  - Conclusion: Yes, the increasing line size reduce the average memory access time from 8.75ns to 8.65ns

- For a system with two levels of cache, define Tc1 = first level cache access time; Tc2 = second level cache access time;  Tm = main memory access time; H1 = first level hit ratio; H2 = combined first/second level hit ratio. Provide an equation for Ta for a read operation.

- Ta=(Tc1 x H1)+(Tc2 x (H2-H1))+(Tm x (1-H2))